Interim Marine Modelling Guidance for Aquaculture Applications

September 2022
This guidance provides information on recommended defaults for marine modelling, including particle tracking of solids and bath medicines.

Alternative approaches to carrying out marine modelling should be outlined and agreed in an appropriate “Method Statement” and be fully detailed in any modelling report.

This guidance provides information on the basic modelling and outputs required. Risks identified from these may trigger the need for additional model output.

For more information on the background modelling processes please refer to:  Regulatory Modelling Guidance for the Aquaculture Sector:  https://www.sepa.org.uk/media/450279/regulatory-modelling-guidance-for-the-aquaculture-sector.pdf

# Bath Medicine Modelling

- Model with a realistic treatment regime for bath medicines (e.g. 4 treatments in 24hrs, with 3hr intervals between treatments).

- In the Screening Report the risk to the wider environment is assessed and should the discharges from multiple farms be identified as a risk SEPA may ask for multiple farms be included in the Marine modelling. Generally, farms are assessed on their individual discharges though.

- Include a run-up period for the hydrodynamics. Once stable, particles can be added

- In these cases, the bathymetry (site depth) and near-bed flowmetry depth in NewDepomod should use the average depth (or the most representative depth) of the seabed underneath the farm rather than the time-averaged deployment depth:

- Calibration: In most cases the model should be calibrated with dye and/ or drogue data. SEPA recommends that dye data are collected in such a way as to allow the comparison of a monitored dye patch with a modelled patch, alongside the calculation of a measure dispersion coefficient. Drogue data are particularly useful in assessing the accuracy of the modelled transport of waste over longer distances, particularly where this influences sensitive features.

- Bath particle parameters should be consistent with those used in calibration runs.

- Length of model runs: The model should be run for each chemical applied for according to the EQSs specified in tables G-1 to G-3 following the final treatment plus an additional 24 hrs to account for concentration peaks. Output is required for the whole time period starting from the beginning of treatment.

- Runs should be undertaken during a period of Spring tides and a period of Neap tides, to ensure the most conservative area and concentration are captured.

- Decay: For Azamethiphos a decay rate of 8.6 days may be applied to the particles. Cypermethrin and Deltamethrin should be modelled without decay. (Nt. The decay rate for Azamethiphos has been updated to 5.6 days as of the October 2020 revision to DEFRA SPC guidance.)

- Bath particles in the top 3m of the water column (or consistent with net depths at treatment, if shallower), should be considered in the output.

- To assess compliance for Azamethiphos after 3hrs, a single release of the 3hr mass should be modelled. A realistic mass should be used, but a passing mass from the short term BathAuto model can be used as a guide to obtain a likely treatment mass, which can be supported by conditions at the site. A plot showing the area above the 250ng/l will be required. Lines should be added to demonstrate the time 3hrs after the first treatment, and the allowed mixing zone area established using BathAuto. The BathAuto mixing zone ellipse is calculated from the mean surface speed, see Annex-G: https://www.sepa.org.uk/media/113498/fish-farm-manual-annex-g.pdf

- To assess the long-term risks from Azamethiphos, a realistic treatment regime should be modelled. A plot showing the area above 40ng/l will be required. Lines should be added to demonstrate the 72hrs after final treatment and the allowed mixing zone area of 0.5km2.

- To assess compliance for Cypermethrin and Deltamethrin after 6hrs, a single release of the 6hr mass should be modelled. A realistic mass should be used, but a passing mass from the short term BathAuto model can be used as a guide to obtain a likely treatment mass, which can be supported by conditions at the site. A plot showing the area above 16ng/l for Cypermethrin and 6ng/l for Deltamethrin will be required. Lines should be added to demonstrate the time 6hrs after the first treatment, and the allowed mixing zone area established using BathAuto. The BathAuto mixing zone ellipse is calculated from the mean surface speed, see Annex-G: https://www.sepa.org.uk/media/113498/fish-farm-manual-annex-g.pdf

- Output should include:

  - Spatial plots at each of the EQS times (see tables G-1 to G-3) and at various time steps in between to demonstrate extent, concentrations and interactions between plumes of the assessed farm and other modelled farms (if identified as required).

  - Time series of maximum concentrations of Azamethiphos for the entire run period.

  - Time series of areal extent at each of the MAC/ EQS concentration contours:

    - 3hr Azamethiphos EQS: area of Azamethiphos concentration >250ng/l over time for the entire run period not bigger than the mixing zone ellipse area calculated in BathAuto.

    - 72hr Azamethiphos EQS: area of Azamethiphos concentration >40 ng/l over time for entire run period not bigger than 0.5km2.

    - 6hr Cypermethrin EQS: area of Cypermethrin concentration >16ng/l over time not bigger than the mixing zone ellipse area calculated in BathAuto.

    - 6hr Deltamethrin EQS: concentration > 6ng/l over time not bigger than the mixing zone ellipse area calculated in BathAuto.

Note: the size of the mixing zone ellipse area for Azamethiphos after 3hrs differs from the size of the mixing zone ellipse area for Cypermethrin and Deltamethrin after 6 hrs, as the area is time dependent.

## When testing for compliance please follow the principles and EQS/ MAC values used in the previous Bath Auto modelling spreadsheet:

Table G-1 Standards for Azamethiphos

| Timescale | Standard | Type |
| --- | --- | --- |
| 3 hours | 250 ng l-1 | EQS |
| 72 hours | 40 ng l-1 | EQS |
| 72 hours | 100 ng l-1 | MAC |

Table G-2 Standard for Cypermethrin

| Timescale | Standard | Type |
| --- | --- | --- |
| 6 hours | 16 ng l-1 | EQS |

Table G-3 Standard for Deltamethrin

| Timescale | Standard | Type |
| --- | --- | --- |
| 6 hours | 6 ng l-1 | EQS |

Please note: Quantities of Cypermethrin, which are demonstrated to be passing the EQS standard described in table G-2 above, will then be reduced by a factor of 267, in order to comply with the Dec 2018 update to the WAT-SG-53 guidance: https://marine.gov.scot/sma/content/supporting-guidance-wat-sg-53-environmental-quality-standards-and-standards-discharges

For additional information on Environmental Quality Standards for bath chemicals please also consult Models for Assessing the use of Medicines in Bath Treatments:  https://www.sepa.org.uk/media/113498/fish-farm-manual-annex-g.pdf

# Solids Modelling

## Solids Waste Modelling

- Farms should be modelled at peak biomass for the entire period. Peak biomass should be used to calculate solids waste, including waste feed and faeces. Recommended default values are:

- Feed requirement = 7kg feed per tonne biomass per day

- Feed Water Percentage = 9%

- Feed Waste Percentage = 3%

- Feed Absorbed Percentage = 85%

- Feed Carbon Percentage = 49%

- Faeces Carbon Percentage = 30%

## Waste Feed Calculation and Default Parameters

Waste solids (kg) = (1 – feed water content) x feed wastage rate x feed load (kg)

Typical values for the water content of the feed and the feed wastage rate are 0.09 (i.e. 9%) and 0.03 (3%) respectively.

Calculating the discharged carbon specifically requires a small addition:

Waste solids (carbon) (kg) = (1 – feed water content) x feed carbon content x feed wastage rate x feed load (kg) where a typical value for the feed carbon content is 0.49 (49%).

Excreted Solids Calculation and Default Parameters:

Excreted solids (kg) = (1 – feed water content) x (1 - feed wastage rate) x (1 - feed absorbed rate) x feed load (kg) where a typical value for the absorption rate is 0.85.

Calculating the excreted carbon specifically requires a small modification:

Excreted solids (carbon) (kg) = (1 – feed water content) x (1 - feed wastage rate) x (1 - feed absorbed rate) x faeces carbon content x feed load (kg) where the faecal carbon content is typical taken to be 0.3 (i.e. 30%).

- All farms included in SEPA screening modelling should be modelled. Farms identified at screening as having the potential to interact should be included at an appropriate model resolution.

- Include a run-up period for the hydrodynamics. Once stable, particles can be added.

- Solids particle parameters should be consistent with those used in calibration runs. If solids are uncalibrated, default parameters should be used:

  - Decay: 0/s

  - Settling Velocity of feed: 0.095m/s (specified directly)

  - Settling Velocity of faeces: 0.032m/s (specified directly)

  - Horizontal Dispersion: 0.1 m2/s

  - Vertical Dispersion: 0.001 m2/s

  - Erosion Threshold: 0.02 N/m2/s

- The model should be run for 365 days (not including the run-up period).

- Runs should start and finish in summer, as a precaution against impacts of storm events, towards the peak deposition period.

- Output should include:

  - Plot showing the extent and concentration of impact, as an average, taken over the last 90 days of the model run at the 250g/m2 contour. (N.B.  This is the approach taken in our default NewDepomod recommendations).

  - Plot of areal extent at the 250g/m2 contour and average concentration, averaged over the last 90 days of the model run included for each farm identified at screening (spatial plot).

  - Include table of area of averaged concentration at the 250g/m2 contour, averaged over the last 90 days of the model run for each farm included identified at screening.

  - Output for both the suspended and deposited solids

# Sensitive Features (Including Other Farms)

Sensitive features may also be highlighted during screening as potentially at risk from bath medicines, solids or both. These should be addressed in the marine modelling.

- Output should include:

  - Plots with features overlaid, showing the extent and concentration of impact, for the various timescales required.

  - Time series of concentration at the sensitive feature point (or polygon centre)

  - Depth profile for each EQS averaged over 90 days for solids and at each EQS for baths, at the sensitive feature point (or polygon centre).

# Model Grid Resolution

The chosen model grid resolution should be fine enough to resolve the cages and sensitive feature points (if applicable).  The maximum grid resolution around the farms should be 50-55m, but a finer resolution down to 25m (the mean is 36m) has shown to be effective in the past. A fine resolution grid resolution is especially important if the flow outputted from the Marine Model is to be used in NewDepomod in conjunction with variable bathymetry.

For information on accessing this document in an alternative format or language please contact SEPA by emailing to equalities@sepa.org.uk

If you are a user of British Sign Language (BSL) the Contact Scotland BSL service gives you access to an online interpreter enabling you to communicate with us using sign language.

http://contactscotland-bsl.org/

| www.sepa.org.uk |  |
| --- | --- |